# UFH CONSOLIDATED BILL OF QUANTITIES - DRAFT

## University of Fort Hare - Conversion of Old Clinic to Finance Department Offices

Project: Alice Campus Finance Offices Conversion

Client: University of Fort Hare

QS: Excel Quantity Surveyors / Dell QS

Prepared by: Claude AI

Date: 2026-01-04

Standard: ASAQS SSM7 (2015) / DQRules v2.0

Status: Draft for Review

__________________________________________________

## SUMMARY OF TRADES

| Bill No. | Trade | Status |
| --- | --- | --- |
| 1 | Preliminaries | Framework - pending customization |
| 2 | Demolitions | Complete |
| 4 | Earthworks | Complete |
| 7 | Concrete, Formwork & Reinforcement | Complete |
| 10 | Masonry | Complete |
| - | Waterproofing | Complete |
| - | Roofing | Complete |
| - | Carpentry & Joinery | Complete |
| - | Ceilings | Complete |
| - | Floor Coverings | Complete |
| - | Wall Finishes & Painting | Complete |
| - | Glazing | Complete |
| - | Metalwork | Complete |
| - | Plumbing & Drainage | Complete |
| - | Electrical Installation | Package - pending integration |
| - | HVAC Installation | Package - pending integration |
| - | Fire Protection | Package - pending integration |
| 22 | External Works | Complete |
| 23 | Provisional Sums | Complete |

__________________________________________________

# BILL NO. 4 - EARTHWORKS

## 4.1 SITE CLEARANCE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.1.1 | Clear site of vegetation, grass and weeds | m2 | 250 |

## 4.2 EXCAVATION

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.2.1 | Excavation to reduce levels for surface bed formation, maximum depth not exceeding 2.00m, in earth | m3 | 61 |
| 4.2.2 | Excavation for strip foundations 600mm wide x 700mm deep, maximum depth not exceeding 2.00m, in earth | m3 | 45 |
| 4.2.3 | Excavation to reduce levels for external paving layerworks, maximum depth not exceeding 500mm, in earth | m3 | 34 |

## 4.3 DISPOSAL

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.3.1 | Disposal of excavated material off site, including loading and carting away | m3 | 140 |

## 4.4 FILLING - INTERNAL

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.4.1 | Filling to make up levels under surface beds with approved crushed stone or G5 material, compacted in layers not exceeding 150mm, average thickness 150mm | m3 | 28 |
| 4.4.2 | Sand blinding over hardcore filling, 50mm thick, levelled and compacted to receive DPM | m2 | 186 |

## 4.5 FILLING - EXTERNAL PAVING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.5.1 | 150mm thick G7 filling compacted to 93% Mod AASHTO density (heavy duty paving areas only) | m3 | 4 |
| 4.5.2 | 150mm thick G5 crusher run sub-base compacted to 98% Mod AASHTO density | m3 | 19 |
| 4.5.3 | 25mm thick sand bedding to paving, levelled and screeded | m2 | 128 |

## 4.6 COMPACTION

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.6.1 | Compaction of formation to receive filling under surface beds | m2 | 186 |
| 4.6.2 | Compaction of insitu subgrade to 90% Mod AASHTO for external paving | m2 | 128 |

## 4.7 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.7.1 | Keep excavations free of water during construction including pumping, temporary drainage and all associated works | Item | 1 |
| 4.7.2 | Provisional Sum for dewatering if extensive pumping required | P.Sum | R15,000 |

__________________________________________________

# BILL NO. 7 - CONCRETE, FORMWORK & REINFORCEMENT

## 7.1 CONCRETE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 7.1.1 | Blinding concrete, 15MPa/19mm, 50mm thick under foundations | m3 | 4 |
| 7.1.2 | Reinforced concrete, 25MPa/26mm, in strip foundations | m3 | 45 |
| 7.1.3 | Reinforced concrete, 25MPa/19mm, in ring beams RB1 (350x450mm) | m3 | 11 |
| 7.1.4 | Reinforced concrete, 25MPa/19mm, in ring beams RB2 (250x250mm) | m3 | 3 |
| 7.1.5 | Reinforced concrete, 30MPa/19mm, in surface beds 125mm thick, power floated finish | m3 | 24 |
| 7.1.6 | Reinforced concrete, 25MPa/19mm, in canopy slab and beam (Provisional) | m3 | 1 |
|  | Total Concrete | m3 | 88 |

## 7.2 FORMWORK

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 7.2.1 | Formwork, rough, to sides of strip foundations | m2 | 150 |
| 7.2.2 | Formwork, smooth (Class F2), to sides and soffits of ring beams | m2 | 114 |
| 7.2.3 | Formwork, smooth, to edges of surface beds 125mm deep | m | 107 |
| 7.2.4 | Formwork, smooth, to canopy slab and beam (Provisional) | m2 | 8 |
|  | Total Formwork | m2 | ~272 |

## 7.3 REINFORCEMENT

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 7.3.1 | High tensile reinforcement to BS4449, including cutting, bending, fixing with spacers and tying wire | t | 5.3 |
| 7.3.2 | Welded steel fabric reinforcement Ref 193, including laps | m2 | 186 |

## 7.4 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 7.4.1 | Concrete test cubes including laboratory testing | No | 60 |
| 7.4.2 | Isolation joints, 10mm wide, to perimeter of surface beds and around columns | m | 120 |
| 7.4.3 | Sawcut joints to surface beds, 3mm wide | m | 80 |

__________________________________________________

# BILL NO. 10 - MASONRY

## 10.1 BLOCKWORK

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.1.1 | Corobrik Breeze Block walling, 14MPa, 230mm thick, in cement mortar Class II, to external walls | m2 | 36 |
| 10.1.2 | Concrete block walling, 7MPa, 230mm thick, in cement mortar Class II, to internal walls | m2 | 76 |
|  | Total Blockwork | m2 | 112 |

## 10.2 FAIR FACE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.2.1 | Extra over blockwork for fair face finish (Platinum Satin) | m2 | 36 |

## 10.3 BEAM FILLING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.3.1 | Filling to underside of ring beams with blockwork | m2 | 22 |

## 10.4 DAMP PROOF COURSE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.4.1 | Malthoid DPC, 230mm wide, under walls | m | 59 |

## 10.5 LINTELS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.5.1 | Fabricated steel lintels, various sizes, over openings | No | 36 |

## 10.6 BRICKFORCE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.6.1 | Brickforce reinforcement, 75mm wide, in every alternate course to loadbearing blockwork | m | 450 |
| 10.6.2 | Brickforce reinforcement, 75mm wide, in every fourth course to non-loadbearing blockwork | m | 300 |

## 10.7 DRYWALL PARTITIONS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.7.1 | Gyproc drywall partitions, 110mm thick, double sided with insulation, complete with all framing, angles and jointing | m2 | 78 |

__________________________________________________

# BILL - WATERPROOFING

## W.1 DAMP PROOF MEMBRANE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| W.1.1 | 250 micron polyethylene DPM, lapped 150mm at joints, under surface beds | m2 | 186 |

## W.2 WET AREA WATERPROOFING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| W.2.1 | Torch-on waterproofing membrane to ablution floors, turned up at edges 100mm | m2 | 26 |
| W.2.2 | Waterproof membrane/coating to ablution walls to 1800mm height | m2 | 63 |

## W.3 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| W.3.1 | Waterproof seal to floor/wall junctions in wet areas | m | 35 |
| W.3.2 | Waterproof seal around floor wastes and pipe penetrations | No | 12 |

__________________________________________________

# BILL - ROOFING

## R.1 ROOF COVERING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.1.1 | Klip-Lok concealed fix roof sheeting, 0.58mm galvanised, colour Safari-boy, fixed to timber purlins including all clips, fasteners and sealants | m2 | 285 |

## R.2 FLASHINGS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.2.1 | Klip-Lok profile flashings, 0.58mm galvanised, colour Safari-boy, including fixings and sealant | m | 106 |

## R.3 RAINWATER GOODS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.3.1 | Seamless aluminium gutter, including brackets at 900mm centres | m | 44 |
| R.3.2 | Extra over for gutter angles | No | 4 |
| R.3.3 | Extra over for gutter stop ends | No | 4 |
| R.3.4 | Extra over for gutter outlets | No | 4 |
| R.3.5 | 80mm diameter aluminium rainwater downpipes, including brackets, bends and connections | m | 13 |
| R.3.6 | Rainwater shoes | No | 4 |
| R.3.7 | Aluminium valley gutter, including fixings | m | 8 |

## R.4 ROOF WATERPROOFING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.4.1 | Torch-on waterproofing membrane to flat roof areas, 2-layer system including primer | m2 | 38 |
| R.4.2 | Extra over for upstands and kerbs | m | 24 |

## R.5 INSULATION

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.5.1 | 135mm glass fibre insulation to roof, laid over ceiling | m2 | 265 |

## R.6 PROVISIONAL SUM

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.6.1 | P.Sum for roof timber structure (trusses, purlins, battens, bracing) | P.Sum | R150,000 |

__________________________________________________

# BILL - CARPENTRY & JOINERY

## CJ.1 DOORS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.1.1 | Timber flush door, 900x2100mm, complete with hardwood frame and ironmongery (hinges, lever handles, mortice lock, stop) | No | 9 |
| CJ.1.2 | Timber flush door, 800x2100mm, complete with hardwood frame and ironmongery | No | 2 |
| CJ.1.3 | Timber flush door, 700x2100mm (WC type), with indicator lock and ironmongery | No | 7 |
| CJ.1.4 | Steel security door, 900x2100mm, to strong rooms, complete with heavy duty frame, multi-point locking system | No | 2 |

## CJ.2 GATES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.2.1 | Steel security gate G01, 1200x2100mm, complete with frame, heavy duty hinges, gate lock, drop bolts | No | 1 |
| CJ.2.2 | Steel security gate G02, 900x2100mm, complete as before | No | 1 |

## CJ.3 BUILT-IN JOINERY

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.3.1 | Security desk unit JT01, complete as drawing A-(74)6001, Melawood carcass, MDF doors, EeziQuartz countertop, including all fittings, locks and finishes | No | 1 |
| CJ.3.2 | Kitchenette unit JT02, complete as drawing A-(74)6002, Melawood carcass, MDF doors and drawers, EeziQuartz countertop and splashback, including all fittings (excluding sink and mixer) | No | 1 |

## CJ.4 SKIRTINGS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.4.1 | Capco Gamma Base Reveal 75 skirting (Type 01), concealed fixed with bonding agent | m | 197 |
| CJ.4.2 | Capco Gamma Flat Skirting 76 (Type 02), concealed fixed with bonding agent | m | 94 |
| CJ.4.3 | Extra over for internal corners | No | 48 |
| CJ.4.4 | Extra over for external corners | No | 12 |
| CJ.4.5 | Extra over for stop ends | No | 36 |

__________________________________________________

# BILL - CEILINGS

## CE.1 SUSPENDED CEILINGS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CE.1.1 | 600 x 600 x 15mm mineral fiber acoustic ceiling tiles on 24mm exposed aluminium T-bar suspension grid system (Ce.01), including all hangers, main runners, cross tees, wall angles and accessories, fixed at 2700mm AFFL - general areas | m2 | 239 |
| CE.1.2 | 600 x 600 x 15mm vinyl faced moisture resistant ceiling tiles on concealed aluminium suspension grid (Ce.02), complete as before - wet areas | m2 | 36 |
|  | Total Suspended Ceiling | m2 | 275 |

## CE.2 BULKHEADS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CE.2.1 | Gypsum plasterboard bulkhead 400mm wide x 300mm drop (Type 01) on light steel framing, jointed and finished ready for painting - perimeter | m | 66 |
| CE.2.2 | Gypsum plasterboard bulkhead 600mm wide x 400mm drop (Type 02) on light steel framing, complete as before - over ring beams | m | 44 |
| CE.2.3 | Gypsum plasterboard bulkhead 300mm wide x 200mm drop (Type 03) on light steel framing, complete as before - internal partitions | m | 35 |
| CE.2.4 | 12.5mm gypsum plasterboard to soffits of bulkheads including jointing and finishing | m2 | 63 |
|  | Total Bulkheads | m | 145 |

## CE.3 ACCESS PANELS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CE.3.1 | 600 x 600mm hinged metal access panel in suspended ceiling including frame and trim | No | 7 |
| CE.3.2 | 450 x 450mm plastic hinged access panel in suspended ceiling including frame and trim | No | 3 |

## CE.4 SHADOW LINES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CE.4.1 | 20mm shadow line detail at wall/ceiling junction formed in suspended ceiling grid system | m | 266 |

__________________________________________________

# BILL - FLOOR COVERINGS

## FL.1 FLOOR TILES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| FL.1.1 | 600 x 600 x 10mm glazed porcelain floor tiles (Fl.01), fixed with tile adhesive, 3mm grouted joints, colour to architect's selection - general areas | m2 | 350 |
| FL.1.2 | 300 x 300 x 9mm anti-slip ceramic floor tiles (Fl.02), fixed with tile adhesive, 3mm grouted joints - wet areas | m2 | 30 |
|  | Total Floor Tiles | m2 | 380 |

## FL.2 FLOOR PREPARATION

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| FL.2.1 | Prepare surface bed to receive floor tiles including grinding and priming | m2 | 380 |

## FL.3 TILE SKIRTINGS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| FL.3.1 | 100mm high coved ceramic tile skirting to match floor tiles (Type 01) | m | 95 |
| FL.3.2 | 100mm high straight ceramic tile skirting (Type 02) | m | 125 |
|  | Total Tile Skirtings | m | 220 |

__________________________________________________

# BILL - WALL FINISHES & PAINTING

## WF.1 PLASTERING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| WF.1.1 | 15mm thick cement plaster (1:4) wood float finish to new blockwork walls internally including dubbing out, rounded internal angles and preparing for painting | m2 | 188 |
| WF.1.2 | Hack off defective plaster and make good to existing walls including dubbing out, feathering edges and preparing for painting | m2 | 40 |
| WF.1.3 | 15mm thick cement plaster to exposed faces of ring beams including rounded arrises and preparing for painting | m2 | 45 |
|  | Total Plastering | m2 | 273 |

## WF.2 WALL TILING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| WF.2.1 | 300 x 600 x 9mm glazed ceramic wall tiles to SANS 1285, fixed with TAL Professional tile adhesive, grouted with TAL Fine Epoxy grout in 3mm joints, colour to architect's selection - ablution walls full height to 2100mm AFFL | m2 | 96 |
| WF.2.2 | Ditto but 600mm high splashback to kitchenette | m2 | 2 |
| WF.2.3 | Aluminium straight edge trim to exposed tile edges including bedding in adhesive | m | 35 |
|  | Total Wall Tiles | m2 | 98 |

## WF.3 PAINTING - WALLS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| WF.3.1 | Prepare, apply 1 coat DULUX Trade Alkali Resistant Primer (full coat) and 2 coats DULUX Trade 100 Low Sheen to new plastered/drywalled walls internally (WF01), colour to architect's selection | m2 | 266 |
| WF.3.2 | Prepare, apply 1 coat DULUX Trade Alkali Resistant Primer (patching) and 2 coats DULUX Trade 100 Low Sheen to existing plastered walls internally (WF02), colour to architect's selection | m2 | 136 |
|  | Total Wall Paint | m2 | 402 |

## WF.4 PAINTING - CEILINGS/BULKHEADS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| WF.4.1 | Prepare, apply 1 coat primer and 2 coats DULUX Trade 100 Low Sheen to gypsum plasterboard bulkhead soffits | m2 | 63 |
| WF.4.2 | Prepare, apply 1 coat primer and 2 coats DULUX Trade 100 Low Sheen to gypsum plasterboard bulkhead sides | m2 | 51 |
|  | Total Ceiling/Bulkhead Paint | m2 | 114 |

__________________________________________________

# BILL - GLAZING

## GL.1 ALUMINIUM WINDOWS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.1.1 | Aluminium window type W01, 600 x 2805mm, side hung, H-SYSTEM flush glazing or equal, 60-80um powder coated finish, complete with friction stays, handles, weatherstripping, 6.38mm clear laminated safety glass | No | 3 |
| GL.1.2 | Aluminium window type W02, 350 x 900mm, side hung opening, complete as before | No | 10 |
| GL.1.3 | Aluminium window type W03, 900 x 1200mm, top hung opening, complete as before | No | 4 |
| GL.1.4 | Aluminium window type W04, 1200 x 1500mm, side hung opening, complete as before | No | 2 |
| GL.1.5 | Aluminium window type W05, 1500 x 1200mm, top hung opening, complete as before | No | 2 |
| GL.1.6 | Aluminium window type W06, 600 x 600mm, fixed glazing, complete as before | No | 7 |
|  | Total Windows | No | 28 |

## GL.2 ALUMINIUM SHOPFRONTS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.2.1 | Aluminium shopfront type SF01, 1275 x 400mm, fixed glazing, complete with 10.38mm laminated safety glass | No | 2 |
| GL.2.2 | Aluminium shopfront type SF02, 2400 x 2400mm, fixed glazing with transom, complete as before | No | 1 |
| GL.2.3 | Aluminium shopfront type SF03, 1800 x 2400mm, single door with sidelight, including door closer, floor spring, handles | No | 2 |
| GL.2.4 | Aluminium shopfront type SF04, 2400 x 2400mm, double doors with sidelights, including closers, floor springs, handles, panic hardware | No | 1 |
| GL.2.5 | Aluminium shopfront type SF05, 1200 x 2400mm, fixed full height glazing | No | 4 |
| GL.2.6 | Aluminium shopfront type SF06, 3000 x 1200mm, counter screen with transaction opening | No | 1 |
| GL.2.7 | Aluminium shopfront type SF07, 1500 x 2100mm, fixed glazing | No | 2 |
| GL.2.8 | Aluminium shopfront type SF08, 900 x 2400mm, fixed full height glazing | No | 3 |
| GL.2.9 | Aluminium shopfront type SF09, 3600 x 2400mm, entrance screen with single door | No | 1 |
|  | Total Shopfronts | No | 17 |

## GL.3 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.3.1 | Approved polysulphide sealant to perimeter of aluminium frames | m | 265 |
| GL.3.2 | Safety manifestation to glass doors and floor-to-ceiling panels to SANS 10400 requirements | m2 | 35 |

## GL.4 PC SUM

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.4.1 | PC Sum for specialist glazing works if glass specification varies | P.Sum | R50,000 |

__________________________________________________

# BILL - METALWORK

## MW.1 GATES (cross-ref with Carpentry)

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.1.1 | Aluminium security gate type G01, 1200 x 2100mm high, powder coated finish, complete with frame, infill panels, heavy duty hinges (3 No.), gate lock, drop bolts, fixed in prepared opening | No | 1 |
| MW.1.2 | Aluminium security gate type G02, 900 x 2100mm high, complete as before | No | 1 |

## MW.2 STRUCTURAL STEEL

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.2.1 | 100 x 100 x 3mm RHS steel post, 2700mm long, hot dip galvanized and powder coated, including base plate, holding down bolts and grouting - shopfront supports | No | 4 |

## MW.3 BOLLARDS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.3.1 | Precast concrete bollard 150mm dia x 900mm high above ground, including 300mm embedment in 300 x 300 x 400mm concrete base grade 25/19, natural finish or painted | No | 9 |

## MW.4 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.4.1 | Galvanized steel frame anchors for aluminium door and window frames | No | 184 |
| MW.4.2 | Heavy duty cast iron manhole cover 600 x 600mm, grade A15, including frame bedded in mortar | No | 2 |

__________________________________________________

# BILL - PLUMBING & DRAINAGE

## PL.1 WC SUITES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.1.1 | Close coupled WC suite comprising vitreous china pan, dual flush cistern (6/3L), seat and cover, complete with flush pipe, soil connector, fixing bolts, all white | No | 2 |
| PL.1.2 | Wall hung WC pan vitreous china with concealed cistern, seat and cover, support frame, flush plate, complete with connections, all white | No | 5 |
| PL.1.3 | Doc M WC suite 480mm high pan, close coupled cistern, seat and cover, complete with grab rails, all white | No | 2 |
|  | Total WC Suites | No | 9 |

## PL.2 URINALS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.2.1 | Wall hung urinal vitreous china with concealed auto-flush system, waste outlet, including spreader and trap, all white | No | 2 |

## PL.3 WASH HAND BASINS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.3.1 | Wall hung wash hand basin 500mm vitreous china with pillar mixer tap, bottle trap, waste, brackets, all white/chrome | No | 2 |
| PL.3.2 | Vanity counter-mounted wash hand basin 450mm vitreous china with deck mixer tap, bottle trap, waste, all white/chrome | No | 4 |
| PL.3.3 | Doc M accessible wash hand basin with lever taps (150mm handles), bottle trap, waste, support brackets, all white/chrome | No | 2 |
|  | Total WHBs | No | 8 |

## PL.4 SINKS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.4.1 | Single bowl stainless steel sink 450x400mm with mixer tap and swivel spout, strainer waste, bottle trap, complete with clips | No | 1 |

## PL.5 COLD WATER PIPEWORK

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.5.1 | 25mm HDPE Class 10 pipe including fittings, clips, sleeves through walls | m | 45 |
| PL.5.2 | 22mm copper Class 0 pipe including fittings, clips, sleeves | m | 35 |
| PL.5.3 | 15mm copper Class 0 pipe including fittings, clips, sleeves | m | 60 |
|  | Total CW Pipework | m | 140 |

## PL.6 HOT WATER PIPEWORK

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.6.1 | 22mm copper Class 0 pipe including fittings, clips, insulation | m | 25 |
| PL.6.2 | 15mm copper Class 0 pipe including fittings, clips, insulation | m | 40 |
|  | Total HW Pipework | m | 65 |

## PL.7 WASTE & VENT PIPES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.7.1 | 110mm uPVC waste pipe including fittings, clips | m | 25 |
| PL.7.2 | 50mm uPVC waste pipe including fittings, clips | m | 35 |
| PL.7.3 | 40mm uPVC waste pipe including fittings, clips | m | 15 |
| PL.7.4 | 110mm uPVC vent pipe through roof including flashing | m | 8 |
| PL.7.5 | 50mm uPVC vent pipe including fittings | m | 12 |
|  | Total Waste/Vent | m | 95 |

## PL.8 VALVES & EQUIPMENT

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.8.1 | 25mm gate valve | No | 1 |
| PL.8.2 | 22mm ball valve | No | 4 |
| PL.8.3 | 15mm angle service valve | No | 20 |
| PL.8.4 | 25mm non-return valve | No | 1 |
| PL.8.5 | 150L electric geyser horizontal with thermostat, drip tray and connections | No | 1 |
| PL.8.6 | Expansion vessel 8L | No | 1 |

## PL.9 BELOW GROUND DRAINAGE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.9.1 | 110mm uPVC below ground drain pipe in trench including bedding | m | 45 |
| PL.9.2 | 160mm uPVC below ground drain pipe in trench including bedding | m | 15 |
| PL.9.3 | Precast concrete inspection chamber 600x450 including cover | No | 3 |
| PL.9.4 | 110mm rodding eye including cover | No | 4 |
| PL.9.5 | 110mm gully complete with grating | No | 2 |
| PL.9.6 | 50L grease trap complete with cover | No | 1 |
|  | Total Drainage | m | 60 |

## PL.10 ACCESSORIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.10.1 | Toilet roll holder stainless steel | No | 9 |
| PL.10.2 | Toilet brush holder stainless steel | No | 9 |
| PL.10.3 | Grab rail 600mm stainless steel | No | 4 |
| PL.10.4 | Mirror 600x450mm with polished edges | No | 8 |
| PL.10.5 | Soap dispenser stainless steel | No | 8 |
| PL.10.6 | Paper towel dispenser stainless steel | No | 6 |
| PL.10.7 | Sanitary bin stainless steel | No | 5 |
| PL.10.8 | Bib tap chrome with hose union | No | 2 |

__________________________________________________

# BILL NO. 22 - EXTERNAL WORKS

## EW.1 SITE CLEARANCE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.1.1 | Strip topsoil average 150mm deep, stockpile on site for reuse | m2 | 150 |
| EW.1.2 | Break up and remove existing concrete hardstanding average 100mm thick, cart away and dispose off site | m2 | 80 |
| EW.1.3 | Clear site of vegetation, grass and weeds | m2 | 150 |

## EW.2 PAVING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.2.1 | 60mm thick Bosun Smooth Ethnic interlocking concrete pavers, charcoal colour, laid on sand bedding in herringbone pattern, joints filled with sand, vibrated and compacted (light duty) | m2 | 104 |
| EW.2.2 | 80mm thick Bosun Smooth Ethnic interlocking concrete pavers, charcoal colour, complete as before (heavy duty) | m2 | 24 |
| EW.2.3 | Cutting pavers to curves, angles and at edges | m | 80 |
|  | Total Paving | m2 | 128 |

## EW.3 EDGING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.3.1 | Precast concrete kerb 200 x 100mm on and including 100mm thick concrete haunching grade 15/19, mortar jointed | m | 65 |
| EW.3.2 | Paving edge restraint including pegs | m | 15 |
|  | Total Edging | m | 80 |

## EW.4 SURFACE DRAINAGE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.4.1 | 100mm wide ACO channel drain including grating, bedded in concrete | m | 12 |
| EW.4.2 | 100mm gully with grating, connection to drain | No | 2 |
| EW.4.3 | 110mm uPVC stormwater drain in trench including bedding | m | 8 |

__________________________________________________

# BILL NO. 23 - PROVISIONAL SUMS & PC ITEMS

## PS.1 PRIME COST SUMS

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.1.1 | PC Sum for landscaping works to external areas including planting, irrigation and establishment maintenance, all to landscape architect's specification when issued | 100,000 |
| PS.1.2 | PC Sum for boundary treatment/fencing allowance as may be directed | 50,000 |
| PS.1.3 | PC Sum for specialist glazing works if glass specification varies from measured | 50,000 |
| PS.1.4 | PC Sum for roof timber structure including trusses, purlins, bracing and installation | 150,000 |
|  | Sub-total PC Sums | 350,000 |

## PS.2 PROVISIONAL SUMS

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.2.1 | Provisional sum for builder's work in connection with electrical installation (2.5% of package value) | TBD |
| PS.2.2 | Provisional sum for builder's work in connection with HVAC installation (2.5% of package value) | TBD |
| PS.2.3 | Provisional sum for builder's work in connection with fire protection installation (2.5% of package value) | TBD |
| PS.2.4 | Provisional sum for dewatering if extensive pumping required | 15,000 |

## PS.3 DAYWORK

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.3.1 | Provisional sum for daywork - labour | 25,000 |
| PS.3.2 | Provisional sum for daywork - plant and equipment | 10,000 |
| PS.3.3 | Provisional sum for daywork - materials | 15,000 |
|  | Sub-total Daywork | 50,000 |

## PS.4 CONTINGENCIES

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.4.1 | Contingency allowance (5% of measured work - to be calculated) | TBD |

__________________________________________________

# SPECIALIST PACKAGES - PENDING INTEGRATION

## Electrical Installation

Source: Electrical Installation\Elec_BOQ_Extracted\251212-UFH Alice Campus-Finance Offices - Elec BOQ Rev00.xlsx

Status: Package complete - awaiting integration

## Fire Detection

Source: Electrical Installation\FD_BOQ_Extracted\251212-UFH Alice Campus-Finance Offices - FD BOQ Rev00 (Contractors).xlsx

Status: Package complete - awaiting integration

## HVAC Installation

Source: Mechanical Installation\HVAC\HVAC_Extracted\P8080M02-BOQ_RevA.xlsx

Status: Package complete - awaiting integration

## Fire Protection

Source: Mechanical Installation\Fire Protection\Fire_Extracted\P8080M07-BOQ.xlsx

Status: Package complete - awaiting integration

## Health & Safety

Source: Health and Safety Spec and BoQ\H and S -BOQ- Conversion Project 20250512.xlsx

Status: Awaiting integration

__________________________________________________

# MEASUREMENT SUMMARY

| Trade | Key Quantities |
| --- | --- |
| Earthworks | Excavation 140m3, Filling 51m3, Compaction 314m2 |
| Concrete | Total 88m3, Formwork 272m2, Rebar 6.0t |
| Masonry | Blockwork 112m2, Drywall 78m2 |
| Waterproofing | DPM 186m2, Ablutions 89m2 |
| Roofing | Sheeting 285m2, Flashings 106m, P.Sum R150k |
| Carpentry | Doors 18No, Gates 2No, Joinery 2No, Skirtings 291m |
| Ceilings | Suspended 275m2, Bulkheads 145m |
| Floor Coverings | Tiles 380m2, Skirtings 220m |
| Wall Finishes | Plaster 273m2, Tiles 98m2, Paint 516m2 |
| Glazing | Windows 28No, Shopfronts 17No (80m2 glass) |
| Metalwork | Gates 2No, Bollards 9No |
| Plumbing | Fittings 20No, Pipework 300m, Drainage 60m |
| External Works | Paving 128m2, Edging 80m |
| PC Sums | R415,000 |

__________________________________________________

Document Status: DRAFT FOR REVIEW

Next Steps:

Integrate specialist packages (Electrical, HVAC, Fire Protection, Fire Detection)

Add H&S BOQ items

Verify all item descriptions against DQRules

Check units of measurement consistency

Add Preliminaries trade

Final QA review

__________________________________________________

Consolidated by: Claude AI

Date: 2026-01-04

Version: Draft 1.0