# UFH DQRules Verification Report

## Tasks UFH-213 & UFH-214

Project: University of Fort Hare - Finance Office Conversion

Date: 2026-01-04

Reference: DQRules v2.0 / ASAQS SSM7 (2015)

__________________________________________________

## UFH-213: ITEM DESCRIPTION VERIFICATION

### EARTHWORKS (Bill 4)

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| 4.1.1 | Site clearance - should state debris, vegetation types | OK | Could add "including grass, weeds, shrubs" |
| 4.2.1 | Excavation - states depth stage (≤2m) | OK | Per DQRule |
| 4.2.2 | Trench excavation - states dimensions | OK | 600x700mm stated |
| 4.2.3 | Paving excavation - states depth | OK | ≤500mm stated |
| 4.3.1 | Disposal - includes loading and carting | OK | Per DQRule |
| 4.4.1 | Filling - states material, compaction method | OK | G5/G7 material, layers stated |
| 4.4.2 | Sand blinding - states thickness | OK | 50mm stated |
| 4.6.1 | Compaction - describes location | OK | "Formation to receive filling" |
| 4.7.1 | Dewatering - comprehensive item description | OK | Includes pumping, drainage |

Earthworks Verdict: COMPLIANT - All descriptions meet DQRules

__________________________________________________

### CONCRETE, FORMWORK & REINFORCEMENT (Bill 7)

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| 7.1.1 | Blinding - strength, aggregate, thickness | OK | 15MPa/19mm, 50mm |
| 7.1.2-4 | Foundations/RB - strength, aggregate stated | OK | 25MPa specified |
| 7.1.5 | Surface beds - strength, thickness, finish | OK | 30MPa, 125mm, power float |
| 7.2.1-4 | Formwork - states rough/smooth, component | OK | Class F2 where appropriate |
| 7.3.1 | Rebar - states standard, includes cutting/bending | OK | BS4449 specified |
| 7.3.2 | Mesh - reference stated, includes laps | OK | Ref 193 |
| 7.4.1 | Test cubes - includes testing | OK | Laboratory testing stated |
| 7.4.2-3 | Joints - widths stated | OK | 10mm, 3mm |

Concrete Verdict: COMPLIANT - Descriptions include strength, dimensions, finishes

Recommended Addition:

Add "spacers and tying wire" to rebar description per DQRules deemed-to-include

__________________________________________________

### MASONRY (Bill 10)

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| 10.1.1 | External blockwork - strength, size, mortar class | OK | 14MPa, 230mm, Class II |
| 10.1.2 | Internal blockwork - strength, size, mortar | OK | 7MPa, 230mm, Class II |
| 10.2.1 | Fair face - as extra over | OK | Per DQRules |
| 10.3.1 | Beam filling - m² as required | OK |  |
| 10.4.1 | DPC - width stated | OK | 230mm Malthoid |
| 10.5.1 | Lintels - fabricated steel | OK | Per DQRule 22 (m or No) |
| 10.6.1-2 | Brickforce - width, course frequency | OK | 75mm, alternate/4th course |
| 10.7.1 | Drywall - thickness, framing included | OK | 110mm, complete with framing |

Masonry Verdict: COMPLIANT - All required information present

__________________________________________________

### WATERPROOFING

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| W.1.1 | DPM - thickness, lap stated | OK | 250 micron, 150mm lap |
| W.2.1 | Floor WP - type, turn-up stated | OK | Torch-on, 100mm turn-up |
| W.2.2 | Wall WP - type, height stated | OK | To 1800mm height |
| W.3.1 | Seals - location stated | OK | Floor/wall junctions |

Waterproofing Verdict: COMPLIANT

__________________________________________________

### ROOFING

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| R.1.1 | Sheeting - type, gauge, colour, fixing | OK | Klip-Lok, 0.58mm, Safari-boy |
| R.2.1 | Flashings - profile, gauge, fixings | OK | Matching specification |
| R.3.1-7 | Rainwater goods - material, size, brackets | OK | Aluminium, spacing stated |
| R.4.1 | Flat roof WP - layers, primer | OK | 2-layer system |
| R.5.1 | Insulation - thickness, type | OK | 135mm glass fibre |

Roofing Verdict: COMPLIANT

__________________________________________________

### CARPENTRY & JOINERY

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| CJ.1.1-3 | Doors - size, material, frame, ironmongery | OK | Complete descriptions |
| CJ.1.4 | Security doors - size, multi-point lock | OK | Strong room spec |
| CJ.2.1-2 | Gates - size, ironmongery listed | OK | Hinges, locks, bolts |
| CJ.3.1-2 | Joinery - drawing ref, materials, finishes | OK | A-(74)6001/6002 referenced |
| CJ.4.1-2 | Skirtings - product name, fixing method | OK | Capco Gamma, concealed fixed |

Carpentry Verdict: COMPLIANT - Drawing references included

__________________________________________________

### CEILINGS

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| CE.1.1 | Suspended ceiling - tile size, grid type, height | OK | 600x600x15, T-bar, 2700mm |
| CE.1.2 | Moisture resistant - distinguished from general | OK | Vinyl faced specified |
| CE.2.1-3 | Bulkheads - dimensions, construction, finish | OK | Width, drop, materials |
| CE.2.4 | Bulkhead soffits - thickness, finish | OK | 12.5mm, ready for painting |
| CE.3.1-2 | Access panels - size, type, frame | OK | Metal/plastic specified |
| CE.4.1 | Shadow line - width stated | OK | 20mm |

Ceilings Verdict: COMPLIANT

__________________________________________________

### FLOOR COVERINGS

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| FL.1.1-2 | Tiles - size, thickness, adhesive, joints | OK | 600x600x10, 3mm grouted |
| FL.2.1 | Preparation - method stated | OK | Grinding and priming |
| FL.3.1-2 | Skirtings - height, type | OK | 100mm coved/straight |

Floor Coverings Verdict: COMPLIANT

__________________________________________________

### WALL FINISHES & PAINTING

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| WF.1.1 | Plaster - thickness, mix, finish | OK | 15mm, 1:4, wood float |
| WF.1.2 | Patch repair - method described | OK | Hack off, make good |
| WF.2.1-2 | Wall tiles - size, adhesive, grout | OK | TAL products specified |
| WF.2.3 | Edge trim - material, fixing | OK | Aluminium, bedded |
| WF.3.1-2 | Paint - prep, primer, coats, product | OK | DULUX Trade specified |
| WF.4.1-2 | Ceiling paint - substrate stated | OK | Plasterboard bulkheads |

Wall Finishes Verdict: COMPLIANT - Product specifications included

__________________________________________________

### GLAZING

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| GL.1.1-6 | Windows - size, type, system, glass | OK | H-SYSTEM, 6.38mm laminated |
| GL.2.1-9 | Shopfronts - size, type, ironmongery | OK | Closers, handles included |
| GL.3.1 | Sealant - type, location | OK | Polysulphide, perimeter |
| GL.3.2 | Manifestation - standard ref | OK | SANS 10400 |

Glazing Verdict: COMPLIANT

__________________________________________________

### METALWORK

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| MW.1.1-2 | Gates - size, finish, ironmongery | OK | Complete with bolts |
| MW.2.1 | Steel posts - size, finish, fixings | OK | Base plate, grouting |
| MW.3.1 | Bollards - size, foundation, finish | OK | Embedment depth stated |
| MW.4.1 | Anchors - material, purpose | OK | Galvanized steel |

Metalwork Verdict: COMPLIANT

__________________________________________________

### PLUMBING & DRAINAGE

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| PL.1.1-3 | WC suites - type, flush, accessories | OK | Complete descriptions |
| PL.2.1 | Urinals - type, flush system | OK | Auto-flush specified |
| PL.3.1-3 | WHBs - type, size, taps, traps | OK | All fittings listed |
| PL.5.1-3 | Pipework - size, material, class | OK | HDPE/Copper specified |
| PL.7.1-5 | Waste/vent - size, material, fittings | OK | uPVC specified |
| PL.9.1-6 | Drainage - size, bedding, chambers | OK | Complete descriptions |
| PL.10.1-8 | Accessories - material, size | OK | Stainless steel |

Plumbing Verdict: COMPLIANT - Comprehensive descriptions

__________________________________________________

### EXTERNAL WORKS (Bill 22)

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| EW.1.1-3 | Site clearance - depth, disposal | OK | 150mm avg, cart away |
| EW.2.1-2 | Paving - type, colour, pattern, thickness | OK | Bosun Smooth Ethnic, charcoal |
| EW.2.3 | Cutting - location stated | OK | Curves, angles, edges |
| EW.3.1 | Kerbs - size, haunching | OK | 200x100, 100mm concrete |
| EW.4.1-3 | Drainage - type, size, bedding | OK | ACO, grating, uPVC |

External Works Verdict: COMPLIANT

__________________________________________________

### PROVISIONAL SUMS (Bill 23)

| Item | Description Check | Status | Notes |
| --- | --- | --- | --- |
| PS.1.1-4 | PC Sums - purpose clearly stated | OK | Scope described |
| PS.2.1-4 | Provisional Sums - scope described | OK | BWIC percentages stated |
| PS.3.1-3 | Daywork - categories separated | OK | Labour, plant, materials |

Provisional Sums Verdict: COMPLIANT

__________________________________________________

## UFH-214: UNITS OF MEASUREMENT VERIFICATION

### Per DQRules - Correct Units by Trade

| Trade | Item Type | Required Unit | BOQ Unit | Status |
| --- | --- | --- | --- | --- |
| Earthworks |  |  |  |  |
|  | Site clearance | m2 | m2 | OK |
|  | Excavation (bulk/trench) | m3 | m3 | OK |
|  | Disposal | m3 | m3 | OK |
|  | Filling | m3 | m3 | OK |
|  | Sand blinding | m2 or m3 | m2 | OK |
|  | Compaction | m2 | m2 | OK |
|  | Dewatering | Item/P.Sum | Item/P.Sum | OK |
| Concrete |  |  |  |  |
|  | Concrete components | m3 | m3 | OK |
|  | Formwork | m2 | m2 | OK |
|  | Formwork to edges | m | m | OK |
|  | Reinforcement bar | t | t | OK |
|  | Fabric reinforcement | m2 | m2 | OK |
|  | Test blocks | No | No | OK |
|  | Joints | m | m | OK |
| Masonry |  |  |  |  |
|  | Blockwork walls | m2 | m2 | OK |
|  | Fair face | m2 | m2 | OK |
|  | Beam filling | m2 | m2 | OK |
|  | DPC | m | m | OK |
|  | Lintels | No | No | OK |
|  | Brickforce | m | m | OK |
|  | Drywall | m2 | m2 | OK |
| Waterproofing |  |  |  |  |
|  | Membranes | m2 | m2 | OK |
|  | Seals | m | m | OK |
|  | Penetrations | No | No | OK |
| Roofing |  |  |  |  |
|  | Sheeting | m2 | m2 | OK |
|  | Flashings | m | m | OK |
|  | Gutters | m | m | OK |
|  | Downpipes | m | m | OK |
|  | Accessories | No | No | OK |
|  | Insulation | m2 | m2 | OK |
| Carpentry |  |  |  |  |
|  | Doors | No | No | OK |
|  | Gates | No | No | OK |
|  | Built-in joinery | No | No | OK |
|  | Skirtings | m | m | OK |
|  | Corners/ends | No | No | OK |
| Ceilings |  |  |  |  |
|  | Suspended ceiling | m2 | m2 | OK |
|  | Bulkheads | m | m | OK |
|  | Bulkhead soffits | m2 | m2 | OK |
|  | Access panels | No | No | OK |
|  | Shadow lines | m | m | OK |
| Floor Coverings |  |  |  |  |
|  | Floor tiles | m2 | m2 | OK |
|  | Tile skirtings | m | m | OK |
|  | Preparation | m2 | m2 | OK |
| Wall Finishes |  |  |  |  |
|  | Plastering | m2 | m2 | OK |
|  | Wall tiling | m2 | m2 | OK |
|  | Edge trim | m | m | OK |
|  | Painting | m2 | m2 | OK |
| Glazing |  |  |  |  |
|  | Windows | No | No | OK |
|  | Shopfronts | No | No | OK |
|  | Sealant | m | m | OK |
|  | Manifestation | m2 | m2 | OK |
| Metalwork |  |  |  |  |
|  | Gates | No | No | OK |
|  | Steel posts | No | No | OK |
|  | Bollards | No | No | OK |
|  | Anchors | No | No | OK |
| Plumbing |  |  |  |  |
|  | Sanitary fittings | No | No | OK |
|  | Pipework | m | m | OK |
|  | Valves | No | No | OK |
|  | Drainage | m | m | OK |
|  | Chambers | No | No | OK |
|  | Accessories | No | No | OK |
| External Works |  |  |  |  |
|  | Site clearance | m2 | m2 | OK |
|  | Paving | m2 | m2 | OK |
|  | Cutting | m | m | OK |
|  | Edging/kerbs | m | m | OK |
|  | Drainage | m | m | OK |
|  | Gullies | No | No | OK |

__________________________________________________

## SUMMARY

### UFH-213: Item Descriptions

Overall Status: COMPLIANT

All item descriptions in the consolidated BOQ meet DQRules requirements:

Dimensions and sizes stated where required

Material specifications included

Mortar classes/concrete strengths specified

Fixing methods described

Relevant standards referenced (SANS, BS)

Product names included where specified

Minor Recommendations:

Add "including grass, weeds, shrubs ≤200mm girth" to site clearance

Add "spacers and tying wire" to rebar descriptions (deemed-to-include)

Add "bedding of roof coverings" to masonry where applicable

### UFH-214: Units of Measurement

Overall Status: COMPLIANT

All units in the consolidated BOQ are correct per DQRules v2.0:

m3 for concrete, excavation, filling

m2 for areas (walls, floors, ceilings, roofing)

m for linear items (skirtings, flashings, pipework)

No for numbered items (doors, windows, fittings)

t for reinforcement

Item/P.Sum for provisional allowances

No corrections required.

__________________________________________________

## VERIFICATION COMPLETE

| Task | Status | Result |
| --- | --- | --- |
| UFH-213 | Complete | COMPLIANT - Minor recommendations only |
| UFH-214 | Complete | COMPLIANT - All units correct |

__________________________________________________

Verified by: Claude AI

Date: 2026-01-04