# UFH-041: BULK EXCAVATION MEASUREMENT

## University of Fort Hare - Conversion of Old Clinic to Finance Offices

Measured by: Claude AI

Date: 2026-01-04

Reference Drawings:

S-01-25055-20-001 Rev E - Foundation Layout

S-01-25055-20-002 Rev E - Surface Bed Layout

A-(11)1000 Rev 4 - Ground Storey Plan

Standard: ASAQS SSM7 (2015) Section 4 - Earthworks

__________________________________________________

## 1. SCOPE OF BULK EXCAVATION

Bulk excavation for this project comprises:

Excavation to formation level for new surface bed construction

Excavation for foundation trenches (measured separately under UFH-042)

Note: Trench excavation for strip foundations (UFH-042) is measured separately and marked complete.

__________________________________________________

## 2. SURFACE BED LAYERWORKS (from S-01-25055-20-002)

| Layer | Thickness |
| --- | --- |
| Concrete surface bed | 125mm |
| DPM membrane | - |
| Sand blinding | 50mm |
| Compacted hardcore/crushed stone | 150mm |
| Total depth to formation | 325mm |

__________________________________________________

## 3. BUILDING AREAS (from Surface Bed Layout)

| Ref | Room | Area (m²) |
| --- | --- | --- |
| 0.01 | Open Plan Office | 33 |
| 0.02 | Open Plan Office (small) | 20 |
| 0.03 | Student Waiting | 15 |
| 0.04 | Enquiries/Client Service Area | 64 |
| 0.07 | Boardroom | 19 |
| 0.11 | Strong Room 1 | 14 |
| 0.12 | Strong Room 2 | 12 |
| 0.14 | Office 4 | 12 |
| 0.15 | Office 5 | 14 |
| 0.17 | Porch | 9 |
| 0.18 | Passage | 24 |
| 0.19 | WC Areas (combined) | 11 |
| 0.20 | Kitchenette | 11 |
| - | Other/circulation | 8 |
|  | TOTAL | 186 m² |

__________________________________________________

## 4. BULK EXCAVATION CALCULATION

### 4.1 Excavation to Reduced Level for Surface Bed Formation

Assumption: Existing floor construction to be removed and new formation level established.

| Description | Calculation | Quantity | Unit |
| --- | --- | --- | --- |
| Surface bed area | Per room schedule | 186 | m² |
| Excavation depth | Per layerworks detail | 0.325 | m |
| Volume | 186 × 0.325 | 60.45 | m³ |
| Say |  | 61 | m³ |

### 4.2 Classification per SSM7

Depth stage: Not exceeding 2.00m (actual depth 325mm)

Material: Assumed normal ground conditions (not rock)

Disposal: To be carted away (conversion project - limited space for stockpiling)

__________________________________________________

## 5. BOQ ITEM FORMAT

### SECTION 4: EARTHWORKS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.1 | EXCAVATION |  |  |
| 4.1.1 | Excavation to reduce levels for surface bed formation, maximum depth not exceeding 2.00m, in earth | m³ | 61 |
| 4.2 | DISPOSAL |  |  |
| 4.2.1 | Disposal of excavated material off site, including loading and carting away | m³ | 61 |

__________________________________________________

## 6. NOTES AND ASSUMPTIONS

Existing floor: Assumed existing floor slab/construction to be demolished and removed (per demolition scope). New excavation required to formation level.

Ground conditions: Normal earth assumed. If rock encountered, additional items required.

Working space: Not measured separately for surface beds (bulk excavation to full area).

Bulking factor: Not applied to excavation volume. Disposal quantity equals excavation quantity for measurement purposes.

Compaction: Compaction of formation measured separately under filling/surface bed items.

Relationship to UFH-042: Trench excavation for strip foundations (approx. 45m³) measured separately under UFH-042 and is additional to this bulk excavation.

__________________________________________________

## 7. QUERY/CLARIFICATION REQUIRED

Q1: Confirm existing floor slab is to be demolished. If existing slab retained, bulk excavation may be reduced to localized areas only.

Q2: Confirm ground conditions - is rock anticipated?

__________________________________________________

## 8. SUMMARY

| Item | Description | Quantity | Unit |
| --- | --- | --- | --- |
| UFH-041 | Bulk excavation to formation for surface beds (≤2m depth) | 61 | m³ |
| UFH-041 | Disposal of excavated material | 61 | m³ |

Total Bulk Excavation Volume: 61 m³

__________________________________________________

Measurement complete: 2026-01-04

Task: UFH-041