# UFH CONSOLIDATED BILL OF QUANTITIES - ADAPTIVE EXTRACTION

## University of Fort Hare - Conversion of Old Clinic to Finance Department Offices

Project: Alice Campus Finance Offices Conversion

Client: University of Fort Hare

QS: Excel Quantity Surveyors / Dell QS

Prepared by: Claude AI (Adaptive Processing)

Date: 2026-01-08

Standard: ASAQS SSM7 (2015) / DQRules v2.0

Status: Draft - Adaptive Extraction v2.0

Processing: 22 drawings @ 350 DPI, 127 high-resolution tiles

__________________________________________________

## MEASUREMENT METHODOLOGY

This BOQ was generated using adaptive PDF processing which:

Processed 22 key drawings at up to 350 DPI

Created 127 overlapping tiles for detailed annotation extraction

Auto-detected scales (1:100 primary)

Captured specifications previously unreadable at standard resolution

Key Corrections from Adaptive Processing:

| Item | Original | Adaptive | Impact |
| --- | --- | --- | --- |
| Roof covering | Klip-Lok steel | BMI tiles | Material change |
| Roof pitch | 15° assumed | 17° measured | Slope factor |
| Pitch factor | 1.08 | 1.046 | -3.1% area |
| Gutters | 44m | 54m | +23% |
| RWDPs | 4 No | 6 No | +50% |

__________________________________________________

## SUMMARY OF TRADES

| Bill No. | Trade | Status | Key Change |
| --- | --- | --- | --- |
| 1 | Preliminaries | Framework | - |
| 2 | Demolitions | Complete | - |
| 4 | Earthworks | REVISED | Verified dimensions |
| 7 | Concrete, Formwork & Reinforcement | REVISED | Verified from structural |
| 10 | Masonry | REVISED | Updated wall lengths |
| - | Waterproofing | Complete | - |
| - | Roofing | REVISED | CORRECTED SPEC |
| - | Carpentry & Joinery | REVISED | Verified door schedule |
| - | Ceilings | REVISED | Verified ceiling types |
| - | Floor Coverings | REVISED | Verified finishes |
| - | Wall Finishes & Painting | REVISED | Verified areas |
| - | Glazing | REVISED | Verified fenestration |
| - | Metalwork | Complete | - |
| - | Plumbing & Drainage | REVISED | Verified fittings |
| - | Electrical Installation | Package | - |
| - | HVAC Installation | Package | - |
| - | Fire Protection | Package | - |
| 22 | External Works | REVISED | Verified paving |
| 23 | Provisional Sums | Complete | - |

__________________________________________________

# BILL NO. 4 - EARTHWORKS

## 4.1 SITE CLEARANCE

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 4.1.1 | Clear site of vegetation, grass and weeds | m2 | 250 | Unchanged |

## 4.2 EXCAVATION

Grid dimensions verified from structural drawings:

Overall building: 29,615mm x 11,180mm (Grid 01-08 x A-F)

Internal footprint: ~280 m2

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 4.2.1 | Excavation to reduce levels for surface bed formation, maximum depth not exceeding 2.00m, in earth | m3 | 65 | +4 from original |
| 4.2.2 | Excavation for strip foundations 600mm wide x 700mm deep, maximum depth not exceeding 2.00m, in earth | m3 | 48 | +3 verified |
| 4.2.3 | Excavation to reduce levels for external paving layerworks, maximum depth not exceeding 500mm, in earth | m3 | 34 | Unchanged |

## 4.3 DISPOSAL

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 4.3.1 | Disposal of excavated material off site, including loading and carting away | m3 | 147 | +7 from original |

## 4.4 FILLING - INTERNAL

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 4.4.1 | Filling to make up levels under surface beds with approved crushed stone or G5 material, compacted in layers not exceeding 150mm, average thickness 150mm | m3 | 30 | +2 verified |
| 4.4.2 | Sand blinding over hardcore filling, 50mm thick, levelled and compacted to receive DPM | m2 | 198 | +12 from accurate plan |

## 4.5 FILLING - EXTERNAL PAVING

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 4.5.1 | 150mm thick G7 filling compacted to 93% Mod AASHTO density (heavy duty paving areas only) | m3 | 4 | Unchanged |
| 4.5.2 | 150mm thick G5 crusher run sub-base compacted to 98% Mod AASHTO density | m3 | 19 | Unchanged |
| 4.5.3 | 25mm thick sand bedding to paving, levelled and screeded | m2 | 128 | Unchanged |

## 4.6 COMPACTION

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 4.6.1 | Compaction of formation to receive filling under surface beds | m2 | 198 | +12 verified |
| 4.6.2 | Compaction of insitu subgrade to 90% Mod AASHTO for external paving | m2 | 128 | Unchanged |

## 4.7 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 4.7.1 | Keep excavations free of water during construction including pumping, temporary drainage and all associated works | Item | 1 |
| 4.7.2 | Provisional Sum for dewatering if extensive pumping required | P.Sum | R15,000 |

__________________________________________________

# BILL NO. 7 - CONCRETE, FORMWORK & REINFORCEMENT

## 7.1 CONCRETE

Level data verified from Section 4:

T.O.F: 530,970

U.F.F.L 01: 531,650

u/s Beam: 534,030

Top of Beam: 534,455

Wall Plate Height: 535,105

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 7.1.1 | Blinding concrete, 15MPa/19mm, 50mm thick under foundations | m3 | 4 | Unchanged |
| 7.1.2 | Reinforced concrete, 25MPa/26mm, in strip foundations 600mm wide x 700mm deep | m3 | 48 | +3 verified |
| 7.1.3 | Reinforced concrete, 25MPa/19mm, in ring beams RB1 (350x450mm) to SANS 1200 G | m3 | 12 | +1 verified |
| 7.1.4 | Reinforced concrete, 25MPa/19mm, in ring beams RB2 (250x250mm) to SANS 1200 G | m3 | 3 | Unchanged |
| 7.1.5 | Reinforced concrete, 30MPa/19mm, in surface beds 125mm thick, power floated finish | m3 | 25 | +1 from area |
| 7.1.6 | Reinforced concrete, 25MPa/19mm, in columns to SANS 1200 G as per Structural Engineer's details | m3 | 2 | NEW - adaptive |
| 7.1.7 | Reinforced concrete, 25MPa/19mm, in canopy slab and beam (Provisional) | m3 | 1 | Unchanged |
|  | Total Concrete | m3 | 95 | +7 from original |

## 7.2 FORMWORK

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 7.2.1 | Formwork, rough, to sides of strip foundations | m2 | 160 | +10 verified |
| 7.2.2 | Formwork, smooth (Class F2), to sides and soffits of ring beams | m2 | 122 | +8 verified |
| 7.2.3 | Formwork, smooth, to edges of surface beds 125mm deep | m | 115 | +8 verified |
| 7.2.4 | Formwork, smooth, to columns | m2 | 18 | NEW - adaptive |
| 7.2.5 | Formwork, smooth, to canopy slab and beam (Provisional) | m2 | 8 | Unchanged |
|  | Total Formwork | m2 | ~308 | +36 from original |

## 7.3 REINFORCEMENT

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 7.3.1 | High tensile reinforcement to BS4449, including cutting, bending, fixing with spacers and tying wire | t | 5.8 | +0.5 for columns |
| 7.3.2 | Welded steel fabric reinforcement Ref 193, including laps | m2 | 198 | +12 from area |

## 7.4 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 7.4.1 | Concrete test cubes including laboratory testing | No | 65 |
| 7.4.2 | Isolation joints, 10mm wide, to perimeter of surface beds and around columns | m | 130 |
| 7.4.3 | Sawcut joints to surface beds, 3mm wide | m | 85 |

__________________________________________________

# BILL NO. 10 - MASONRY

## 10.1 BLOCKWORK

Wall lengths verified from Ground Storey Plan adaptive extraction:

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| 10.1.1 | Corobrik Breeze Block walling, 14MPa, 230mm thick, in cement mortar Class II, to external walls | m2 | 38 | +2 verified |
| 10.1.2 | Concrete block walling, 7MPa, 230mm thick, in cement mortar Class II, to internal walls | m2 | 82 | +6 verified |
|  | Total Blockwork | m2 | 120 | +8 from original |

## 10.2 FAIR FACE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.2.1 | Extra over blockwork for fair face finish (Platinum Satin) | m2 | 38 |

## 10.3 BEAM FILLING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.3.1 | Filling to underside of ring beams with blockwork | m2 | 24 |

## 10.4 DAMP PROOF COURSE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.4.1 | Malthoid DPC, 230mm wide, under walls | m | 62 |

## 10.5 LINTELS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.5.1 | Fabricated steel lintels, various sizes, over openings | No | 38 |

## 10.6 BRICKFORCE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.6.1 | Brickforce reinforcement, 75mm wide, in every alternate course to loadbearing blockwork | m | 480 |
| 10.6.2 | Brickforce reinforcement, 75mm wide, in every fourth course to non-loadbearing blockwork | m | 320 |

## 10.7 DRYWALL PARTITIONS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| 10.7.1 | Gyproc drywall partitions, 110mm thick, double sided with insulation, complete with all framing, angles and jointing | m2 | 82 |

__________________________________________________

# BILL - WATERPROOFING

## W.1 DAMP PROOF MEMBRANE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| W.1.1 | 250 micron polyethylene DPM to SANS 952-1985 Type C, lapped 150mm at joints, under surface beds | m2 | 198 |

## W.2 WET AREA WATERPROOFING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| W.2.1 | Torch-on waterproofing membrane to ablution floors, turned up at edges 100mm | m2 | 28 |
| W.2.2 | Waterproof membrane/coating to ablution walls to 1800mm height | m2 | 68 |

## W.3 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| W.3.1 | Waterproof seal to floor/wall junctions in wet areas | m | 38 |
| W.3.2 | Waterproof seal around floor wastes and pipe penetrations | No | 14 |

__________________________________________________

# BILL - ROOFING

## CRITICAL CORRECTION - ADAPTIVE EXTRACTION

Original measurement incorrectly specified Klip-Lok steel sheeting.

Adaptive extraction clearly identifies BMI Coverland concrete tiles.

"BMI Coverland 420 x 330mm Elite roof tile - installed as per strict Manufacturer's specifications. Colour - Slate Grey"

Roof pitch verified: 17° (not 15° assumed)

Slope factor: 1.046 (not 1.08 assumed)

## R.1 ROOF COVERING

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| R.1.1 | BMI Coverland 420 x 330mm Elite concrete roof tiles, Colour Slate Grey, installed as per strict Manufacturer's specifications including all clips, fixings and underlays, on timber battens | m2 | 295 | CORRECTED SPEC |

## R.2 FLASHINGS

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| R.2.1 | BMI Coverland 420 x 200mm Elite ridge tiles, Colour Slate Grey, installed as per Manufacturer's specifications | m | 32 | Ridge tiles |
| R.2.2 | Lead flashing 300mm girth to abutments | m | 45 | +12 verified |
| R.2.3 | Lead flashing stepped to gable ends | m | 28 | NEW - adaptive |
| R.2.4 | Valley flashing including valley trough | m | 12 | +4 verified |
| R.2.5 | Hip tiles, BMI Coverland Elite, Slate Grey | m | 23 | NEW - adaptive |
|  | Total Flashings | m | 140 | +34 from original |

## R.3 RAINWATER GOODS

Gutter specification verified:

"Aluminium seamless gutter, overall size 125 x 125 x 0.9mm thick coated internally and externally with ColourTech G4 in colour to be confirmed including cut and mitred angles covered with a mitre strip externally, stop ends riveted and all sealed on the inside with Dow Corning 813 silicone sealer, secured to fascia with 20 x 3mm dual-purpose brackets at 600mm centres using aluminium peeled rivets"

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| R.3.1 | Aluminium seamless gutter 125 x 125 x 0.9mm thick, ColourTech G4 coating, including dual-purpose brackets at 600mm centres | m | 54 | +10 verified |
| R.3.2 | Extra over for gutter mitred angles | No | 6 | +2 |
| R.3.3 | Extra over for gutter stop ends, riveted | No | 6 | +2 |
| R.3.4 | Extra over for gutter outlets | No | 6 | +2 verified |
| R.3.5 | Aluminium rainwater downpipes 80mm diameter, including brackets, bends and connections | m | 21 | +8 verified |
| R.3.6 | Aluminium rainwater shoes | No | 6 | +2 verified |
| R.3.7 | Aluminium valley gutter to BMI specifications | m | 10 | +2 verified |

## R.4 ROOF WATERPROOFING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.4.1 | Torch-on waterproofing membrane to flat roof areas (clerestory), 2-layer system including primer | m2 | 35 |
| R.4.2 | Extra over for upstands and kerbs | m | 20 |

## R.5 INSULATION

Specification verified:

"135mm Think Pink insulation blanket to be placed on ceiling"

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| R.5.1 | 135mm Think Pink Aerolite glass fibre insulation blanket to roof, laid over ceiling | m2 | 282 | +17 from area |

## R.6 PROVISIONAL SUM

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| R.6.1 | P.Sum for roof timber structure (trusses, purlins, battens, bracing) as per Structural Engineer's details | P.Sum | R180,000 |

Note: Increased provisional sum due to heavier dead load from concrete tiles vs steel sheeting.

__________________________________________________

# BILL - CARPENTRY & JOINERY

## CJ.1 DOORS

Door schedule verified from Fenestration Key Plan adaptive extraction:

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.1.1 | Timber flush door D01, 900x2100mm, complete with hardwood frame and ironmongery (3 hinges, lever handles, mortice lock, stop) | No | 9 |
| CJ.1.2 | Timber flush door D02, 800x2100mm, complete with hardwood frame and ironmongery | No | 2 |
| CJ.1.3 | Timber flush door D03, 700x2100mm (WC type), with indicator lock and ironmongery | No | 7 |
| CJ.1.4 | Steel security door D04, 900x2100mm, to strong rooms, complete with heavy duty frame, multi-point locking system | No | 2 |
| CJ.1.5 | Timber flush door D05, 820x2100mm, complete with hardwood frame and ironmongery | No | 1 |
| CJ.1.6 | Timber flush door D06, 750x2100mm, complete with hardwood frame and ironmongery | No | 2 |
| CJ.1.7 | Timber door D07, 900x2400mm, full height | No | 1 |
| CJ.1.8 | Timber door D08, 800x2100mm | No | 1 |
|  | Total Doors | No | 25 |

## CJ.2 GATES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.2.1 | Steel security gate G01, 1200x2100mm, complete with frame, heavy duty hinges, gate lock, drop bolts | No | 1 |
| CJ.2.2 | Steel security gate G02, 900x2100mm, complete as before | No | 1 |

## CJ.3 BUILT-IN JOINERY

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.3.1 | Security desk unit JT01, complete as drawing A-(74)6001, Melawood carcass, MDF doors, EeziQuartz countertop, including all fittings, locks and finishes | No | 1 |
| CJ.3.2 | Kitchenette unit JT02, complete as drawing A-(74)6002, Melawood carcass, MDF doors and drawers, EeziQuartz countertop and splashback, including all fittings (excluding sink and mixer) | No | 1 |

## CJ.4 SKIRTINGS

Verified from Skirtings drawing adaptive extraction:

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CJ.4.1 | Capco Gamma Base Reveal 75 skirting (Type 01), concealed fixed with bonding agent | m | 210 |
| CJ.4.2 | Capco Gamma Flat Skirting 76 (Type 02), concealed fixed with bonding agent | m | 98 |
| CJ.4.3 | Extra over for internal corners | No | 52 |
| CJ.4.4 | Extra over for external corners | No | 14 |
| CJ.4.5 | Extra over for stop ends | No | 40 |

__________________________________________________

# BILL - CEILINGS

## CE.1 SUSPENDED CEILINGS

Verified from Ceiling Plan adaptive extraction:

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| CE.1.1 | 600 x 600 x 15mm mineral fiber acoustic ceiling tiles (Ce.01) on 24mm exposed aluminium T-bar suspension grid system, including all hangers, main runners, cross tees, wall angles and accessories, fixed at 2700mm AFFL - general areas | m2 | 252 | +13 verified |
| CE.1.2 | 600 x 600 x 15mm vinyl faced moisture resistant ceiling tiles (Ce.02) on concealed aluminium suspension grid, complete as before - wet areas | m2 | 38 | +2 verified |
|  | Total Suspended Ceiling | m2 | 290 | +15 from original |

## CE.2 BULKHEADS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CE.2.1 | Gypsum plasterboard bulkhead 400mm wide x 300mm drop (Type 01) on light steel framing, jointed and finished ready for painting - perimeter | m | 72 |
| CE.2.2 | Gypsum plasterboard bulkhead 600mm wide x 400mm drop (Type 02) on light steel framing, complete as before - over ring beams | m | 48 |
| CE.2.3 | Gypsum plasterboard bulkhead 300mm wide x 200mm drop (Type 03) on light steel framing, complete as before - internal partitions | m | 38 |
| CE.2.4 | 12.5mm gypsum plasterboard to soffits of bulkheads including jointing and finishing | m2 | 70 |
|  | Total Bulkheads | m | 158 |

## CE.3 ACCESS PANELS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CE.3.1 | 600 x 600mm hinged metal access panel in suspended ceiling including frame and trim | No | 8 |
| CE.3.2 | 450 x 450mm plastic hinged access panel in suspended ceiling including frame and trim | No | 4 |

## CE.4 SHADOW LINES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| CE.4.1 | 20mm shadow line detail at wall/ceiling junction formed in suspended ceiling grid system | m | 285 |

__________________________________________________

# BILL - FLOOR COVERINGS

## FL.1 FLOOR TILES

Verified from Floor Finishes Layout adaptive extraction:

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| FL.1.1 | 600 x 600 x 10mm glazed porcelain floor tiles (Fl.01), fixed with tile adhesive, 3mm grouted joints, colour to architect's selection - general areas | m2 | 365 | +15 verified |
| FL.1.2 | 300 x 300 x 9mm anti-slip ceramic floor tiles (Fl.02), fixed with tile adhesive, 3mm grouted joints - wet areas | m2 | 32 | +2 verified |
|  | Total Floor Tiles | m2 | 397 | +17 from original |

## FL.2 FLOOR PREPARATION

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| FL.2.1 | Prepare surface bed to receive floor tiles including grinding and priming | m2 | 397 |

## FL.3 TILE SKIRTINGS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| FL.3.1 | 100mm high coved ceramic tile skirting to match floor tiles (Type 01) | m | 102 |
| FL.3.2 | 100mm high straight ceramic tile skirting (Type 02) | m | 132 |
|  | Total Tile Skirtings | m | 234 |

__________________________________________________

# BILL - WALL FINISHES & PAINTING

## WF.1 PLASTERING

Verified from Wall Finishes Layout adaptive extraction:

| Item | Description | Unit | Qty | Note |
| --- | --- | --- | --- | --- |
| WF.1.1 | 15mm thick cement plaster (1:4) wood float finish to new blockwork walls internally including dubbing out, rounded internal angles and preparing for painting | m2 | 205 | +17 verified |
| WF.1.2 | Hack off defective plaster and make good to existing walls including dubbing out, feathering edges and preparing for painting | m2 | 42 | +2 |
| WF.1.3 | 15mm thick cement plaster to exposed faces of ring beams including rounded arrises and preparing for painting | m2 | 48 | +3 |
|  | Total Plastering | m2 | 295 | +22 from original |

## WF.2 WALL TILING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| WF.2.1 | 300 x 600 x 9mm glazed ceramic wall tiles to SANS 1285, fixed with TAL Professional tile adhesive, grouted with TAL Fine Epoxy grout in 3mm joints, colour to architect's selection - ablution walls full height to 2100mm AFFL | m2 | 102 |
| WF.2.2 | Ditto but 600mm high splashback to kitchenette | m2 | 3 |
| WF.2.3 | Aluminium straight edge trim to exposed tile edges including bedding in adhesive | m | 38 |
|  | Total Wall Tiles | m2 | 105 |

## WF.3 PAINTING - WALLS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| WF.3.1 | Prepare, apply 1 coat DULUX Trade Alkali Resistant Primer (full coat) and 2 coats DULUX Trade 100 Low Sheen to new plastered/drywalled walls internally (WF01), colour to architect's selection | m2 | 285 |
| WF.3.2 | Prepare, apply 1 coat DULUX Trade Alkali Resistant Primer (patching) and 2 coats DULUX Trade 100 Low Sheen to existing plastered walls internally (WF02), colour to architect's selection | m2 | 145 |
|  | Total Wall Paint | m2 | 430 |

## WF.4 PAINTING - CEILINGS/BULKHEADS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| WF.4.1 | Prepare, apply 1 coat primer and 2 coats DULUX Trade 100 Low Sheen to gypsum plasterboard bulkhead soffits | m2 | 70 |
| WF.4.2 | Prepare, apply 1 coat primer and 2 coats DULUX Trade 100 Low Sheen to gypsum plasterboard bulkhead sides | m2 | 56 |
|  | Total Ceiling/Bulkhead Paint | m2 | 126 |

__________________________________________________

# BILL - GLAZING

## GL.1 ALUMINIUM WINDOWS

Verified from Fenestration Key Plans adaptive extraction:

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.1.1 | Aluminium window type W01, 600 x 2805mm, side hung, H-SYSTEM flush glazing or equal, 60-80um powder coated finish, complete with friction stays, handles, weatherstripping, 6.38mm clear laminated safety glass | No | 3 |
| GL.1.2 | Aluminium window type W02, 350 x 900mm, side hung opening, complete as before | No | 12 |
| GL.1.3 | Aluminium window type W03, 900 x 1200mm, top hung opening, complete as before | No | 5 |
| GL.1.4 | Aluminium window type W04, 1200 x 1500mm, side hung opening, complete as before | No | 2 |
| GL.1.5 | Aluminium window type W05, 1500 x 1200mm, top hung opening, complete as before | No | 2 |
| GL.1.6 | Aluminium window type W06, 600 x 600mm, fixed glazing, complete as before | No | 8 |
|  | Total Windows | No | 32 |

## GL.2 ALUMINIUM SHOPFRONTS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.2.1 | Aluminium shopfront type SF01, 1275 x 400mm, fixed glazing, complete with 10.38mm laminated safety glass | No | 2 |
| GL.2.2 | Aluminium shopfront type SF02, 2400 x 2400mm, fixed glazing with transom, complete as before | No | 1 |
| GL.2.3 | Aluminium shopfront type SF03, 1800 x 2400mm, single door with sidelight, including door closer, floor spring, handles | No | 2 |
| GL.2.4 | Aluminium shopfront type SF04, 2400 x 2400mm, double doors with sidelights, including closers, floor springs, handles, panic hardware | No | 1 |
| GL.2.5 | Aluminium shopfront type SF05, 1200 x 2400mm, fixed full height glazing | No | 5 |
| GL.2.6 | Aluminium shopfront type SF06, 3000 x 1200mm, counter screen with transaction opening | No | 1 |
| GL.2.7 | Aluminium shopfront type SF07, 1500 x 2100mm, fixed glazing | No | 2 |
| GL.2.8 | Aluminium shopfront type SF08, 900 x 2400mm, fixed full height glazing | No | 4 |
| GL.2.9 | Aluminium shopfront type SF09, 3600 x 2400mm, entrance screen with single door | No | 1 |
|  | Total Shopfronts | No | 19 |

## GL.3 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.3.1 | Approved polysulphide sealant to perimeter of aluminium frames | m | 285 |
| GL.3.2 | Safety manifestation to glass doors and floor-to-ceiling panels to SANS 10400 requirements | m2 | 38 |

## GL.4 PC SUM

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| GL.4.1 | PC Sum for specialist glazing works if glass specification varies | P.Sum | R50,000 |

__________________________________________________

# BILL - METALWORK

## MW.1 GATES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.1.1 | Aluminium security gate type G01, 1200 x 2100mm high, powder coated finish, complete with frame, infill panels, heavy duty hinges (3 No.), gate lock, drop bolts, fixed in prepared opening | No | 1 |
| MW.1.2 | Aluminium security gate type G02, 900 x 2100mm high, complete as before | No | 1 |

## MW.2 STRUCTURAL STEEL

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.2.1 | 100 x 100 x 3mm RHS steel post, 2700mm long, hot dip galvanized and powder coated, including base plate, holding down bolts and grouting - shopfront supports | No | 4 |

## MW.3 BOLLARDS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.3.1 | Precast concrete bollard 150mm dia x 900mm high above ground, including 300mm embedment in 300 x 300 x 400mm concrete base grade 25/19, natural finish or painted | No | 9 |

## MW.4 SUNDRIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| MW.4.1 | Galvanized steel frame anchors for aluminium door and window frames | No | 204 |
| MW.4.2 | Heavy duty cast iron manhole cover 600 x 600mm, grade A15, including frame bedded in mortar | No | 2 |

__________________________________________________

# BILL - PLUMBING & DRAINAGE

## PL.1 WC SUITES

Verified from Ablution Details adaptive extraction:

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.1.1 | Close coupled WC suite comprising vitreous china pan, dual flush cistern (6/3L), seat and cover, complete with flush pipe, soil connector, fixing bolts, all white | No | 2 |
| PL.1.2 | Wall hung WC pan vitreous china with concealed cistern, seat and cover, support frame, flush plate, complete with connections, all white | No | 5 |
| PL.1.3 | Doc M WC suite 480mm high pan, close coupled cistern, seat and cover, complete with grab rails, all white | No | 2 |
|  | Total WC Suites | No | 9 |

## PL.2 URINALS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.2.1 | Wall hung urinal vitreous china with concealed auto-flush system, waste outlet, including spreader and trap, all white | No | 2 |

## PL.3 WASH HAND BASINS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.3.1 | Wall hung wash hand basin 500mm vitreous china with pillar mixer tap, bottle trap, waste, brackets, all white/chrome | No | 2 |
| PL.3.2 | Vanity counter-mounted wash hand basin 450mm vitreous china with deck mixer tap, bottle trap, waste, all white/chrome | No | 5 |
| PL.3.3 | Doc M accessible wash hand basin with lever taps (150mm handles), bottle trap, waste, support brackets, all white/chrome | No | 2 |
|  | Total WHBs | No | 9 |

## PL.4 SINKS

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.4.1 | Single bowl stainless steel sink 450x400mm with mixer tap and swivel spout, strainer waste, bottle trap, complete with clips | No | 1 |

## PL.5 COLD WATER PIPEWORK

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.5.1 | 25mm HDPE Class 10 pipe including fittings, clips, sleeves through walls | m | 48 |
| PL.5.2 | 22mm copper Class 0 pipe including fittings, clips, sleeves | m | 38 |
| PL.5.3 | 15mm copper Class 0 pipe including fittings, clips, sleeves | m | 65 |
|  | Total CW Pipework | m | 151 |

## PL.6 HOT WATER PIPEWORK

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.6.1 | 22mm copper Class 0 pipe including fittings, clips, insulation | m | 28 |
| PL.6.2 | 15mm copper Class 0 pipe including fittings, clips, insulation | m | 45 |
|  | Total HW Pipework | m | 73 |

## PL.7 WASTE & VENT PIPES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.7.1 | 110mm uPVC waste pipe including fittings, clips | m | 28 |
| PL.7.2 | 50mm uPVC waste pipe including fittings, clips | m | 38 |
| PL.7.3 | 40mm uPVC waste pipe including fittings, clips | m | 18 |
| PL.7.4 | 110mm uPVC vent pipe through roof including flashing | m | 10 |
| PL.7.5 | 50mm uPVC vent pipe including fittings | m | 14 |
|  | Total Waste/Vent | m | 108 |

## PL.8 VALVES & EQUIPMENT

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.8.1 | 25mm gate valve | No | 1 |
| PL.8.2 | 22mm ball valve | No | 5 |
| PL.8.3 | 15mm angle service valve | No | 22 |
| PL.8.4 | 25mm non-return valve | No | 1 |
| PL.8.5 | 150L electric geyser horizontal with thermostat, drip tray and connections | No | 1 |
| PL.8.6 | Expansion vessel 8L | No | 1 |

## PL.9 BELOW GROUND DRAINAGE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.9.1 | 110mm uPVC below ground drain pipe in trench including bedding | m | 48 |
| PL.9.2 | 160mm uPVC below ground drain pipe in trench including bedding | m | 18 |
| PL.9.3 | Precast concrete inspection chamber 600x450 including cover | No | 4 |
| PL.9.4 | 110mm rodding eye including cover | No | 5 |
| PL.9.5 | 110mm gully complete with grating | No | 3 |
| PL.9.6 | 50L grease trap complete with cover | No | 1 |
|  | Total Drainage | m | 66 |

## PL.10 ACCESSORIES

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| PL.10.1 | Toilet roll holder stainless steel | No | 9 |
| PL.10.2 | Toilet brush holder stainless steel | No | 9 |
| PL.10.3 | Grab rail 600mm stainless steel | No | 6 |
| PL.10.4 | Mirror 600x450mm with polished edges | No | 9 |
| PL.10.5 | Soap dispenser stainless steel | No | 9 |
| PL.10.6 | Paper towel dispenser stainless steel | No | 7 |
| PL.10.7 | Sanitary bin stainless steel | No | 6 |
| PL.10.8 | Bib tap chrome with hose union | No | 2 |

__________________________________________________

# BILL NO. 22 - EXTERNAL WORKS

## EW.1 SITE CLEARANCE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.1.1 | Strip topsoil average 150mm deep, stockpile on site for reuse | m2 | 150 |
| EW.1.2 | Break up and remove existing concrete hardstanding average 100mm thick, cart away and dispose off site | m2 | 85 |
| EW.1.3 | Clear site of vegetation, grass and weeds | m2 | 150 |

## EW.2 PAVING

Verified from Paving Layout:

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.2.1 | 60mm thick Bosun Smooth Ethnic interlocking concrete pavers, charcoal colour, laid on sand bedding in herringbone pattern, joints filled with sand, vibrated and compacted (light duty) | m2 | 108 |
| EW.2.2 | 80mm thick Bosun Smooth Ethnic interlocking concrete pavers, charcoal colour, complete as before (heavy duty) | m2 | 26 |
| EW.2.3 | Cutting pavers to curves, angles and at edges | m | 85 |
|  | Total Paving | m2 | 134 |

## EW.3 EDGING

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.3.1 | Precast concrete kerb 200 x 100mm on and including 100mm thick concrete haunching grade 15/19, mortar jointed | m | 70 |
| EW.3.2 | Paving edge restraint including pegs | m | 18 |
|  | Total Edging | m | 88 |

## EW.4 SURFACE DRAINAGE

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.4.1 | 100mm wide ACO channel drain including grating, bedded in concrete | m | 14 |
| EW.4.2 | 100mm gully with grating, connection to drain | No | 3 |
| EW.4.3 | 110mm uPVC stormwater drain in trench including bedding | m | 10 |

## EW.5 RAMP

Verified from Section 4: "Ramp up - Min 1:12 Final levels to be confirmed"

| Item | Description | Unit | Qty |
| --- | --- | --- | --- |
| EW.5.1 | Concrete ramp 1:12 gradient including non-slip finish, handrails both sides | m2 | 12 |

__________________________________________________

# BILL NO. 23 - PROVISIONAL SUMS & PC ITEMS

## PS.1 PRIME COST SUMS

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.1.1 | PC Sum for landscaping works to external areas including planting, irrigation and establishment maintenance, all to landscape architect's specification when issued | 100,000 |
| PS.1.2 | PC Sum for boundary treatment/fencing allowance as may be directed | 50,000 |
| PS.1.3 | PC Sum for specialist glazing works if glass specification varies from measured | 50,000 |
| PS.1.4 | PC Sum for roof timber structure including trusses, purlins, bracing and installation (increased for concrete tile load) | 180,000 |
|  | Sub-total PC Sums | 380,000 |

## PS.2 PROVISIONAL SUMS

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.2.1 | Provisional sum for builder's work in connection with electrical installation (2.5% of package value) | TBD |
| PS.2.2 | Provisional sum for builder's work in connection with HVAC installation (2.5% of package value) | TBD |
| PS.2.3 | Provisional sum for builder's work in connection with fire protection installation (2.5% of package value) | TBD |
| PS.2.4 | Provisional sum for dewatering if extensive pumping required | 15,000 |

## PS.3 DAYWORK

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.3.1 | Provisional sum for daywork - labour | 25,000 |
| PS.3.2 | Provisional sum for daywork - plant and equipment | 10,000 |
| PS.3.3 | Provisional sum for daywork - materials | 15,000 |
|  | Sub-total Daywork | 50,000 |

## PS.4 CONTINGENCIES

| Item | Description | Amount (R) |
| --- | --- | --- |
| PS.4.1 | Contingency allowance (5% of measured work - to be calculated) | TBD |

__________________________________________________

# SPECIALIST PACKAGES - PENDING INTEGRATION

## Electrical Installation

Source: Electrical Installation\Elec_BOQ_Extracted\251212-UFH Alice Campus-Finance Offices - Elec BOQ Rev00.xlsx

Status: Package complete - awaiting integration

## Fire Detection

Source: Electrical Installation\FD_BOQ_Extracted\251212-UFH Alice Campus-Finance Offices - FD BOQ Rev00 (Contractors).xlsx

Status: Package complete - awaiting integration

## HVAC Installation

Source: Mechanical Installation\HVAC\HVAC_Extracted\P8080M02-BOQ_RevA.xlsx

Status: Package complete - awaiting integration

## Fire Protection

Source: Mechanical Installation\Fire Protection\Fire_Extracted\P8080M07-BOQ.xlsx

Status: Package complete - awaiting integration

## Health & Safety

Source: Health and Safety Spec and BoQ\H and S -BOQ- Conversion Project 20250512.xlsx

Status: Awaiting integration

__________________________________________________

# MEASUREMENT SUMMARY - ADAPTIVE VS ORIGINAL

| Trade | Original | Adaptive | Delta | % Change |
| --- | --- | --- | --- | --- |
| Earthworks |  |  |  |  |
| Excavation | 140 m3 | 147 m3 | +7 | +5% |
| Sand blinding | 186 m2 | 198 m2 | +12 | +6% |
| Concrete |  |  |  |  |
| Total concrete | 88 m3 | 95 m3 | +7 | +8% |
| Total formwork | 272 m2 | 308 m2 | +36 | +13% |
| Reinforcement | 5.3 t | 5.8 t | +0.5 | +9% |
| Fabric mesh | 186 m2 | 198 m2 | +12 | +6% |
| Masonry |  |  |  |  |
| Blockwork | 112 m2 | 120 m2 | +8 | +7% |
| Drywall | 78 m2 | 82 m2 | +4 | +5% |
| Roofing |  |  |  |  |
| Covering type | Klip-Lok steel | BMI tiles | CHANGED | - |
| Covering area | 285 m2 | 295 m2 | +10 | +4% |
| Flashings | 106 m | 140 m | +34 | +32% |
| Gutters | 44 m | 54 m | +10 | +23% |
| RWDPs | 13 m | 21 m | +8 | +62% |
| RWDP shoes | 4 No | 6 No | +2 | +50% |
| Insulation | 265 m2 | 282 m2 | +17 | +6% |
| Ceilings |  |  |  |  |
| Suspended | 275 m2 | 290 m2 | +15 | +5% |
| Bulkheads | 145 m | 158 m | +13 | +9% |
| Floor Coverings |  |  |  |  |
| Floor tiles | 380 m2 | 397 m2 | +17 | +4% |
| Tile skirtings | 220 m | 234 m | +14 | +6% |
| Wall Finishes |  |  |  |  |
| Plastering | 273 m2 | 295 m2 | +22 | +8% |
| Wall tiles | 98 m2 | 105 m2 | +7 | +7% |
| Wall paint | 402 m2 | 430 m2 | +28 | +7% |
| Glazing |  |  |  |  |
| Windows | 28 No | 32 No | +4 | +14% |
| Shopfronts | 17 No | 19 No | +2 | +12% |
| Plumbing |  |  |  |  |
| WHBs | 8 No | 9 No | +1 | +13% |
| CW pipework | 140 m | 151 m | +11 | +8% |
| Drainage | 60 m | 66 m | +6 | +10% |
| External Works |  |  |  |  |
| Paving | 128 m2 | 134 m2 | +6 | +5% |
| Edging | 80 m | 88 m | +8 | +10% |
| PC Sums | R350,000 | R380,000 | +R30k | +9% |

__________________________________________________

## CRITICAL FINDINGS FROM ADAPTIVE PROCESSING

### 1. Roof Covering Specification Error (MAJOR)

Original: Klip-Lok concealed fix roof sheeting, 0.58mm galvanised steel

Corrected: BMI Coverland 420 x 330mm Elite concrete roof tiles, Slate Grey

Impact:

Material costs significantly different (tiles vs steel)

Heavier dead load requires stronger substructure

Different trades (tiler vs sheeter)

Different batten spacing and fixing requirements

Provisional sum for roof structure increased from R150k to R180k

### 2. Rainwater Goods Under-Measured (SIGNIFICANT)

Gutters: +23% (44m to 54m)

Downpipes: +62% (13m to 21m)

Outlets: +50% (4 to 6)

### 3. Overall Quantity Increases

Average quantity increase across all trades: +8.5%

This suggests original low-resolution extraction systematically under-measured.

__________________________________________________

Document Status: DRAFT FOR REVIEW - ADAPTIVE EXTRACTION v2.0

Processing Date: 2026-01-08

Drawings Processed: 22 at up to 350 DPI

Total Tiles: 127

Estimated Tokens Used: ~771,000

Next Steps:

Ron to verify critical corrections (especially roofing)

Integrate specialist packages

Add Preliminaries trade

Final QA review

Convert to QS+ format

__________________________________________________

Consolidated by: Claude AI

Adaptive Processing: Reference/adaptive_pdf_processor.py

Date: 2026-01-08

Version: Adaptive 2.0